Jaemie’s Notes on Restoration and Refit of Australia II Mini
Editing by Rod Thomas

The Ben Lexcen Mini 12 was designed by Ben in 1986 and an estimated 20 to 50 boats  were built in Western Australia prior to the Fremantle 1987 Americas Cup Races. They were designed as a training boat and for hire fleets with very basic fitout.

Scale is 1/4 of the original Australia II 
Dimensions:  length 4.8m, beam 93cm and draft 90cm.
Mast length is 5.8 m (keel stepped mast)

“ I purchased three bare hulls with masts and not much else from Royal South Australian Yacht Squadron where they had been out of the water and unused for many years.

The first of the three hulls was restored in mid 2024. I trialled my rigging design based on a modern 2.4mR class rig with influences from Mirror, Fireball and Dragon class rigs. The second hull I sold to a guy from Adelaide. 

The third boat I fully restored over 8 months from mid 2025. The restoration included converting the rig so that 2.4mR sails could be used by using a mast 40cm shorter than the mast that came with the boat. I used a cut down Fireball Boom and fittings from a damaged Fireball class boat. I made a Genoa from an old Dragon sail. I use a Fireball spinnaker and 2.4mR mainsail, with a 2.4mR jib for windy weather. 
Steering is via foot pedals located behind the mast.

I made a launching trolley from an aluminium Jetski trolley.

The hull is solid fibreglass and the deck a plywood core with fibreglass skins.  The deck is bolted to the deck via a flange. There was some damage to the inside of a section of the hull that I repaired with epoxy glue and fibreglassed over, The deck was not well bonded in a few locations so I drilled out and  re-bolted in a few spots as well as putting in epoxy plugs and epoxy filleting along the deck and hull joint.

 I also made the shelve across the hull for all the controls from a combination of fibreglass sheathed tasmanian oak timber as well as a section of foam sandwich. This panel I then fibreglassed and bolted to chain stays with I also beefed up with foam and fibreglass.

The hull was faired with filler and sanded before applying Norglass high gloss enamel with a small foam roller.I sanded the deck and tinted the green paint for the deck using a syringe Norglass gloss cream and green paint taking the gloss off by adding silica. The keel was camouflaged like the original Australia II with blue paint. 

As a final step the Hull was wet and dried with 1200 grit sandpaper and cut and polished and waxed twice. I had stickers made for the stripes and lettering and my son Hamish designed the kangaroos for the bow. “

A video of the Rally featuring the Australia II Mini1 12 produced by Jaemie and his son Alex is at https://www.youtube.com/watch?v=q8HaDfnvm20.

There are other videos on the 2026 Rally on You Tube including an overview at 
https://www.youtube.com/watch?v=gfY5rMpNBnU